| 551 277 01 |  |
| --- | --- |
| # PD 3950 USB to P-NET Interface ## General information The PD 3950 USB to P-NET Interface module provides an interface between the P-NET Fieldbus and a standard PC running VIGO The Fieldbus Management System version 5.7 or higher. Using the PD 3950, any standard desktop, laptop or single board PC with a USB port, may be used as a node in a P-NET installation, in applications such as process and factory automation, machine control, building automation, data acquisition etc. |  |
| ## Specific Features  P-NET Fieldbus communication interface.  P-NET RS485, multi-master P-NET interface.  Galvanically isolated P-NET communication port.  Manual baudrate selection 9600, 76800 Baud.  Can be connected to any standard USB port or USB HUB.  Powered by the USB port via the USB cable.  Supports firmware upgrade via USB.  Channel for holding application licences.  Compact designed, suitable for wall mounting, mounting underneath desk etc. ## PD 3950 Block Schematic The following block diagram shows the internal structure of a PD 3950 USB to P-NET Interface: ## Channel Structure The PD 3950 consists of 3 channels as shown in the table. Channel No.; Channel name; Channel description. 0; Service; Service channel. 1; ##### LicenceCh; Channel for holding application licences. 2; ##### USBCh; USB channel. ## Electrical Specifications Power consumption; Max. 246 mA. Temperature. Operating temperature; -25 °C - 70 °C. Storage temperature; -40 °C - 85 °C. Humidity. Relative humidity:; Max. 95%. EMC. EN 61000-6-2, EN 61000-6-3. ## Mechanical Details The PD 3950 USB Interface is housed in a black plastic case. |
| Dimensions; (WxHxD); 66 x 51 x 35 mm. Case material; Polycarbonate. Front foil; ABS. Back plate; Black anodized aluminum. Weight; 75 grams. |
| ## USB Cable PROCES-DATA recommends that only the provided USB cable is used together with the PD 3950 USB interface. |  |